CPE301 – SPRING 2019

# Design Assignment X

Student Name: Ivan Soto

Student #: 2000921825

Student Email:  sotoi2@unlv.nevada.edu

Primary Github address: https://github.com/sotoi2/submission_da

Directory:ESD301/DA4

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Insert initial code here

/*

* DA4A.c

* Author : Ivan

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

volatile unsigned int motor;

char check = 0;

void adc_init(void);

int main(void)

{

DDRB = (1<<1);

DDRC |= (1<<1);//button

PORTB |= (1<<0);

TCCR1A |= (1<<COM1A1)|(1<<COM1B1)|(1<<WGM11); //set timers

TCCR1B |= (1<<WGM13)|(1<<WGM12)|(1<<CS11);    // to correct configurations,

ICR1 = 10000; // for pwm timer

PORTC |= (1<<1);  // make portc1 and output

PCICR= (1<< PCIE1);

PCMSK1 = (1<<PCINT9); //interrupt mask

sei();  // enable interrupts

while(1){} //poll

}

void adc_init(void)

{

// set the adc with appropriate value to registers

DIDR0=0x01;

ADMUX = (1<<REFS0);

ADCSRA |= (1<<ADEN) |

(1<<ADPS2) |

(1<<ADPS1) |

(1<<ADPS0);

ADCSRB = 0x0;

}

ISR(PCINT1_vect)

{

if(!(PINC & (1<<PINC1)))// poll for the switch

{

if(check ==1) //when toggled, turn on the motor and turn

{

PORTB |= (1<<1);

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF)) == 0);

motor = ADC & 0x03FF;

OCR1A = 10*motor;

}

else if (check == 0) // turn off motor when not toggled

{

OCR1A = 0;  //reset count timer value

PORTB &= ~(1<<1);

}

check ^= 1;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Insert only the modified sections here

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

unsuccesful

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA https://github.com/sotoi2/submission_da/blob/master/ESD301/DA4A/IvanSoto4A.docx

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT