CPE301 – SPRING 2019

# Design Assignment X

Student Name: Ivan Soto

Student #: 2000921825

Student Email: sotoi2@unlv.nevada.edu

Primary Github address: https://github.com/sotoi2/submission_da

Directory:ESD301/DA2A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Assembly Language Code:

Insert initial code here

;

; DA2AT1ASM.asm

; Author : Ivan

;

; Replace with your application code

.INCLUDE "M328pDEF.INC"

SBI DDRB, 2  // This sets the bit 2 of PORTB as an output

L1:

SBI PORTB, 2 // This will set the second bit in PORTB

call delay1// Here is the delay for the assembly code for (435 ms)

CBI PORTB, 2// This will clear the second bit in PORTB

call delay2// Here is the delay for the assembly code for (290 ms)

jmp L1 // Continue doing this foreveer

delay1:

//Here is my delay code for 435 ms (6.96 million cycles)

LDI R17, 255

L2: LDI R18, 255

L3: LDI R19, 21

L4: NOP

NOP

DEC R19

BRNE L4

DEC R18

BRNE L3

DEC R17

BRNE L2

ret

delay2:

//Here is my delay code for 290 ms (4.64 million cycles)

LDI R16, 255

L5: LDI R20, 255

L6: LDI R21, 21

L7: NOP

NOP

DEC R21

BRNE L7

DEC R20

BRNE L6

DEC R16

BRNE L5

ret

C Code:

/*

* Author : Ivan

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1 << 2); // This will set bit 2 of PORTB.

while (1)

{

PORTB |= (1 << 2 );

_delay_ms(435);

PORTB |= (1 << 2);

_delay_ms(290);

}

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Insert only the modified sections here

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Code shows a waveform generated with 60% DC and 0.725 second period.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT