CPE301 – SPRING 2019

# Design Assignment 2C

Student Name: Ivan Soto

Student #: 2000921825

Student Email: sotoi2@unlv.nevada.edu

Primary Github address: https://github.com/sotoi2/submission_da

Directory: ESD301/DA2C

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Insert initial code here

/*

* DA2CT1.c

* Author : Ivan

*/

// Implement Design Assignment 2A using Timer 0  normal mode. Count OVF

//occurrence if needed. Do not use interrupts.

//Task one was making a wavorm of 60% duty cycle.

//T2 was button light

#define F_CPU 16000000UL

#include <avr/io.h>

int main() {

uint8_t Count = 0; //  counter

TCNT0 = 0x0; //rReset timer

DDRB |= ( 1<< 2);  //PORTB.2 as output

TCCR0A = 0; //Normal Operation

TCCR0B = (1<<CS02)|(1<<CS00); // Set prescalerto 1024 start timer

while (1){

while((TIFR0 & 0x01) == 0)

{

}

Count++; // Increment counter

if(Count < 25)//25 cycles for the wave , 45 for on

{

TCNT0 = 0x0; // reset timer

TIFR0 = 0x01; // clear flag

}

if(Count == 25)

{

PORTB &= ~(1 << 2); // make PINB.2 low

}

if((25 < Count)&  (Count < 45))

{

TCNT0 = 0x0; // Reset timer

TIFR0 = 0x01; // clear flag, no interrupts

}

if(Count == 45)

{

PORTB |= (1<<2); //  PINB.2 is high

Count = 0; //reset the counter

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Insert only the modified sections here

/*

* DA2CT1-2.c

*

*

* Author : Ivan

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

uint8_t count = 0; // counter

DDRC &= (0<< 2); // set pinc1 as input

PORTC |= (1<<2); // enable pull up

DDRB |= (1<<2); // set portb.2 as output

TCNT0 = 0x2; // reset timer

TCCR0B = (1<<CS02)|(1<<CS00); // set prescalar

while (1)

{

count = 0; // no counting for no press

PORTB |= (1<<2); //turn off led

if(!(PINC & (1<< PINC1))) // poll for switch input

{

TCNT0 = 0x3; // reset timer

PORTB &= ~(1<<2); // turn on led

while(count < 75)

{

while((TIFR0 & 0x01) == 0)

{

}

count++; // increment count with flag

if(count < 75)

{

TCNT0 = 0x3; // reset timer

TIFR0 = 0x01; // clear Timer

}

}

}

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

V

- VIDEO LINKS OF EACH DEMO

youtu.be/o_2E3shbyO0

- GITHUB LINK OF THIS DA  -

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ivan Soto