CPE301 – SPRING 2019

# Design Assignment X

Student Name: Ivan Soto

Student #: 2000921825

Student Email: sotoi2@unlv.nevada.edu

Primary Github address: https://github.com/sotoi2/submission_da

Directory: ESD301/DA2C

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Insert initial code here

/*

* DA2CT2.c

*

*

* Author : Ivan

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int main(void){

DDRB |= (1<< 2); //DDRB.2 IS AN OUTPUT

TIMSK0 |= (1<<TOIE0); // MASK

TCNT0 = 55; // value for our needed wave

sei(); // global interrupt

TCCR0B |= (1<< CS02)|(1<<CS00); // Set prescalar to 1024 and starts timer

while(1)

{

//poll

}

}

// Interrupt C Code

ISR(TIMER0_OVF_vect) // // overflow

{

int Count=0; // new count

Count = Count+1;

if(Count < 25)//25 cycles for the wave , 45 for on

{

TCNT0 = 0x0; // reset timer

TIFR0 = 0x01; // clear flag

}

if(Count == 25)

{

PORTB &= ~(1 << 2); // make PINB.2 low

}

if((25 < Count)&  (Count < 45))

{

TCNT0 = 0x0; // Reset timer

TIFR0 = 0x01; // clear flag, no interrupts

}

if(Count == 45)

{

PORTB |= (1<<2); //  PINB.2 is high

Count = 0; //reset counter

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

/*

* DA2CT2-2.c

*

* Author : Ivan

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int counter;

int main(void){

counter = 0; // set counter to 0

DDRB |= (1<< 2);// portb.2 is an output

TIMSK0 |= (1<<TOIE0); // enable int flag

TCNT0 = 55; // set value to 55 (start from there)

sei(); // global interrupts

TCCR0B |= (1<< CS02)|(1<<CS00); // prescalar to 1024

while(1){

PORTB |= (1<<2); //TURN OFF LED

if(!(PINC & (1 << PINC1))) // PINC IS SWITCH

{

counter = 0; // set to 0 at start of press

TCNT0 = 0x2; // reset our timer

PORTB &= ~(1<< 2); // turn on led

while(counter < 75)

{

}}}}

eISR(TIMER0_OVF_vect) // the interrupt

{

counter++; // increment counter

int count;

count = counter;

if(count < 75){

TCNT0 = 0x2; // Reset timer

TIFR0 = 0x01; // clear timer

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

V

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA  - https://github.com/sotoi2/submission_da/blob/master/ESD301/DA2C/DA2CT2/IvanSoto2C-2.docx

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Ivan Soto