CPE301 – SPRING 2019

# Design Assignment X

Student Name: Ivan Soto

Student #: 2000921825

Student Email: sotoi2@unlv.nevada.edu

Primary Github address: https://github.com/sotoi2/submission_da

Directory: ESD301/DA2C

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

- /*

- * DA2CT3.c

- *

- * Author : Ivan

- */

- #define F_CPU 16000000UL

- #include <avr/io.h>

- #include <avr/interrupt.h>

- int counter; //counter  variable

- int main(void)

- {

- counter = 0; // set counter to 0

- DDRB |= (1<<2); // port b is output

- OCR0A = 250; // value to compare

- TCCR0A |= (1<<WGM01); // ctc mode

- TIMSK0 |= (1<<OCIE2A); // for our interrupts and comparison

- TCCR0B = 0b101; //prescalar is 1024

- sei(); // allow interrupts

- while(1)

- {

- //poll

- }

- }

- ISR(TIMER0_COMPA_vect){

- counter++; // increment the counteer

- int count; // to use in theinterrupt

- count = counter; // set to count for the amount

- if(count == 25) {// time for the delay

- PORTB &= ~(0<<2 ); //PINB.2 on

- }

- if(count == 45) {

- PORTB |= (1<<2); // pinb2 is off

- count = 0; // reset counter

- }

- }

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- /*

- * DA2CT3-2.c

- *

- * Author : Ivan

- */

- #include <avr/io.h>

- #define F_CPU 16000000UL

- #include <avr/interrupt.h>

- int counter; // make a counter variable

- int main(void)

- {

- counter = 0; // set counter to 0

- DDRB |= (1 <<2 ); // make portb.2 an output

- DDRC &= ( 0 << 1); // make pinc.1 an input

- PORTC |= (1<<1);// enable pull up

- OCR0A = 250; // comparison value

- TCCR0A |= (1<<WGM01); // ctc mode

- TIMSK0 |= (1<<OCIE2A); // interrupt when value is reached

- TCCR0B = 0b101; // prescalar to 1024

- sei(); //for global int

- while(1)

- {PORTB&= ~(1<<2);

- PORTB |= (1<< 2); //TURN OFF LED

- if(!(PINC & (1<< PINC1)))//  POLL for switch

- {

- counter = 0; // reset counter

- TCNT0 = 0x0; //reset the timer

- PORTB&= ~(1<<2); // turn on led

- while(counter > 75)

- {

- }

- }

- }

- }

- ISR(TIMER0_COMPA_vect){

- counter++;

- }

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

V

- VIDEO LINKS OF EACH DEMO

youtu.be/OJVMInUMza4

- GITHUB LINK OF THIS DA  -

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT